Smart Ultrasonic Water Meter V1

## Product Requirements Specification (PRS)

Document Version: 1.0
Product Name: Smart Ultrasonic Water Meter V1
Prepared By: Ramisetty Ranganadh
Target Release: V1 Engineering Prototype

# 1. Purpose

The Smart Ultrasonic Water Meter V1 shall accurately measure water consumption using ultrasonic transit-time technology and communicate meter data wirelessly using LoRaWAN. The system shall be designed for long operational life, low maintenance, remote diagnostics, OTA firmware updates, and modular field serviceability.

# 2. Product Objectives

The product shall:

- Measure water flow without moving mechanical parts.

- Achieve utility-grade metering accuracy.

- Operate for 10 years on a replaceable battery.

- Support a total service life of 20 years.

- Allow field repair within 2 hours.

- Support LoRaWAN-based remote monitoring.

- Support OTA firmware updates.

- Detect and report abnormal operating conditions.

# 3. Product Scope

### Included in V1

- DN15 ultrasonic water meter

- Transit-time ultrasonic measurement

- LoRaWAN communication

- Totalized volume measurement

- Instantaneous flow rate measurement

- Battery health monitoring

- Event logging

- Leak detection

- Reverse flow detection

- Tamper detection

- OTA firmware update

- Self-diagnostics

### Excluded from V1

- Mobile application

- NB-IoT connectivity

- Cellular communication

- BLE commissioning

- AI-based predictive analytics

- GPS tracking

# 4. System Overview

Ultrasonic Transducers

│

▼

MSP430FR6047

│

┌────────┼─────────┐

│ │ │

▼ ▼ ▼

FRAM SX1262 Battery

Storage LoRaWAN Monitor

│

▼

Gateway

│

▼

Cloud

# 5. Functional Requirements

## FR-001 Flow Measurement

The meter shall measure water flow using ultrasonic transit-time technology.

### Acceptance Criteria

- Measure forward flow.

- Measure reverse flow.

- No moving mechanical parts.

## FR-002 Volume Calculation

The system shall calculate cumulative water consumption.

### Acceptance Criteria

- Volume stored in non-volatile memory.

- Data retained after power loss.

- Data retained after battery replacement.

## FR-003 Flow Rate Measurement

The system shall provide instantaneous flow rate measurement.

### Units

Liters/minute (LPM)

## FR-004 Leakage Detection

The meter shall detect abnormal continuous water flow.

### Alarm Criteria

Example:

Flow > 0.1 LPM

for > 8 Hours

Event shall be logged.

## FR-005 Reverse Flow Detection

The system shall detect reverse water flow.

### Response

- Generate alarm

- Log event

- Include status in LoRa packet

## FR-006 Tamper Detection

The system shall detect:

- Cover opening

- Magnetic tampering

- Unauthorized access

## FR-007 Battery Monitoring

The system shall monitor:

- Battery voltage

- Remaining life estimate

- Battery replacement event

## FR-008 Event Logging

The system shall maintain event history.

Minimum events:

Power ON

Power Reset

Low Battery

Leak Alarm

Reverse Flow

Tamper

OTA Update

Battery Replacement

Transducer Replacement

## FR-009 Wireless Communication

The system shall communicate using LoRaWAN.

### Transmitted Data

Meter ID

Flow Rate

Volume

Battery Status

Alarm Status

Firmware Version

## FR-010 OTA Firmware Upgrade

The system shall support remote firmware updates.

Requirements:

- Dual-image architecture

- CRC verification

- Rollback support

## FR-011 Self Diagnostics

The meter shall perform health monitoring.

### Diagnostics

- RAM test

- FRAM test

- RTC verification

- Battery status

- Radio status

- Ultrasonic signal quality

# 6. Non-Functional Requirements

## NFR-001 Service Life

Target Life: 20 Years

## NFR-002 Battery Life

Target Life: 10 Years

Battery shall be replaceable.

## NFR-003 Serviceability

Field repair time:

≤ 2 Hours

## NFR-004 Reliability

24 Hours x 365 Days Operation

with automatic recovery after power interruptions.

## NFR-005 Data Integrity

Critical data shall be protected using:

CRC32

Redundant Storage

## NFR-006 Maintainability

Field-replaceable modules:

- Battery pack

- Electronics board

- Transducer cartridge

# 7. Hardware Requirements

## MCU

MSP430FR6047

Responsibilities:

- Ultrasonic metrology

- Power management

- Diagnostics

- Communications control

## LoRa Module

SX1262

## Storage

External FRAM

Stores:

- Volume

- Calibration

- Event logs

- Configuration

## Battery

ER26500

- 3.6V Lithium Thionyl Chloride

- Replaceable

## Flow Tube

DN15

Material:

- Brass or Stainless Steel

- Replaceable transducer cartridge

# 8. Firmware Architecture

Bootloader

|

|--> Image Validation

|--> OTA Upgrade

|--> Rollback

|

Application

|

|-->Meter Manager

|-->Flow Manager

|-->Battery Manager

|-->Event Logger

|-->Alarm Manager

|-->LoRa Manager

|-->Calibration Manager

|-->Diagnostics Manager

|-->Health Monitor

# 9. Operating States

INIT

POST

IDLE

MEASURE

PROCESS

LOG

TRANSMIT

FAULT

# 10. Alarm Definitions

| Alarm ID | Description |
| --- | --- |
| A001 | Low Battery |
| A002 | Leak Detected |
| A003 | Reverse Flow |
| A004 | Tamper Detected |
| A005 | RTC Failure |
| A006 | Radio Failure |
| A007 | Transducer Failure |
| A008 | Calibration Error |

# 11. Serviceability Requirements

## Battery Replacement

Target Time < 10 Minutes

## Transducer Replacement

Target Time < 15 Minutes

## Electronics Replacement

Target Time < 30 Minutes

# 12. Success Criteria for V1

The V1 prototype shall be considered successful when it can:

✅ Measure water flow accurately

✅ Calculate total water consumption

✅ Detect leaks

✅ Detect reverse flow

✅ Store data in FRAM

✅ Transmit meter data via LoRaWAN

✅ Support OTA firmware update

✅ Operate from battery power

✅ Support modular replacement of battery and transducer assembly